Comparison FSG 70/71M <<===>> AR6201 for gliders

| FSG70/71M | AR20XX Function | AR6201 Pin No.: | AR6201 Designation | AR6201 Function |  |
| --- | --- | --- | --- | --- | --- |
| 11 | -Battery | P1-13 | P_SUPP_GND | Supply Voltage Ground |  |
| 12 | -Battery | P1-25 | P_SUPP_GND | Supply Voltage Ground |  |
| 9 | + Battery | P1-11 | P_SUPP | Supply voltage |  |
| 10 | + Battery | P1-12 | P_SUPP | Supply voltage |  |
| 11 | Audio Mass | P1-14 | SPK_LO | Mass |  |
| 2 | Microphone Ground | P1-6 and | MIKE_DYN_LO | Input Dynamic Microphone, Low, Balanced |  |
|  |  | Connect P1-8 | MIKE_STD_LO | Mass |  |
| 1 | Microphone Input | P1-5 | MIKE_DYN_HI | Input Dynamic Microphone, High, Balanced |  |
| 7 | Send button | P1-17 | /PTT | Send button |  |
| 15 | Loudspeaker | P1-1 | SPK_HI | Speaker output, asymmetrical |  |
| if headbuilders are connected to 14. , to distinguish them between the P1-2 connect and P1-3 is connected to ground. |  | P1-2 | HDPH1_A | Headphone Output 1, Balanced | Optional |
|  |  | P1-3 | HDPH1_B | Headphone Output 1, Balanced |  |
| 3 | Standard microphone | P1-18 | MIKE_STD_HI | Input for Standard Aviation Microphones |  |
| 5 | Intercom | P1-7 | /IC | Activation Intercom |  |
| 4 | NF-Fremd I | P1-4 | AF_AUX_IN_HI | Only needed when transmitting external audio signals. Must be enabled in setup. |  |
| 8 | Lighting 14 V | P1-23 | ILL_HI | Only required for adjustable lighting and must be activated in the setup. |  |